# O’tega Iteku & 20482233

# Software Writing Robot Manual

# Software Description

This project intends to design a robotic system which takes the desire output of a user, written in English text, and have a robot write down what the user has typed into written, legible, English. To elablorate further, the user’s input will be stored in a sperate file called “TextData.txt” with a specified font for each text, stored in a separate file called  “SingleStrokeFont.txt”, this is then transferred as commands to a virtual port called “RS3232”, which leads to the Arduino MEGA unit, turning the commands into readable code for the robotic system to execute it’s intended function.

Furthermore, there are specific parameters that the code must fulfil;
1. The text created by the robotic system must be contained inside a 100mm box

2. Said text from the user’s input must be between 4mm to 10mm tall

Problems arise however that need to be solved for this project to be successful. Firstly, in the SingleStrokeFont.txt file, a group of functions must be developed to offset the individual letters in the x axis direction and y axis direction using the LF (ASCII 10) and CR  (ASCII 13), is to perfectly offset lines, but also a spacing of 5mm between each line. 

In addition, after developing the font through the SinlgeStrokeFont.txt file, it must be converted into G code for the writing arm to understand the commands, this is done through a virtual serial port (RS3232), where the writing arm almost forms a “queue” of G commands, taking each one by one, this being called a buffer.

# Project Files

TextScale.h – Contains the implementation and prototype of the TextScale function, which takes the user’s input of the text height, and then converts that into a scale for the text. The conditional compilation to be met is the user’s input must be between 4.0mm to 10mm, if not the function will automatically set the user input to 10mm. The only parameters are the TextScale.h file and <stdio.h> file.

TextScale.h – Contains the prototypes and similar parameters from the .c file

LoadFontMap.c – Contains the implementation and prototype of the LoadFontMap function, the function loads the file “SingleStrokeFont.txt” (If it cannot load this file, it will output an error message), and is stores the file in FontMap array with the Font Data struct, with the max number of variables stored being 1024, from the MAX_STROKES declaration. After every “999” marker in the SingleStrokeFont.txt file,  the FontMap stores the ascii character (the second character) and in the embedded struct called penData, stores the number of strokes need to write that character, if the SingleStrokeFont.txt file is malformed (e.g contains to more than one character in a line), then an error message will output. The conditional compilation are the TotalLoadedChar and the FontMap definition, and the MAX_STROKES declaration. Parameters are the <stdio.h> file and the LoadFontMap.h file

LoadFontMap.h – Contains the prototype of the LoadFontMap Function, contains the definitions for the PenData and FontData structs, external declaration of the TotalLoadedChar variable and external declaration of the FontMap with the FontData struct with the limited variables constrained to the MAX_STROKES (At 1024). The parameters used are header guards (#ifndef and #endif) which prevent the file declaration from being repeated multiple times.

WordIntoGCode.c – Contains the prototype and implementation of the WordIntoGCode function, this function finds the corresponding ASCII character of a letter given to it (In this case it would be the testfile.txt letters) from the FontMap, if this letter cannot be found in the SingleStrokeFont.txt file the function will skip it with a warning. Afterwards, the function will take the strokes from the character and implement as G Code, scale to the user’s textScale, and then updates the pen position. The conditional compilations are the FontMap array must not be malformed, and the foundIndex (Variables which becomes the pointer for the FontMap  Array) must not be equal to -1. Parameters are the <stdio.h> file, <stdlib.h> file, “rs232.h” file, “serial.h” file, "textScale.h” file, "LoadFontMap.h” file, "ProcessTextFile.h” file, "WordIntoGCode” file.

WordIntoGCode.h – Contains the prototype of the WordIntoGCode function. The conditional compilation for this file that it requires the prototype declarations of the textScale and external declaration of FontMap array in the FontData struct with the MAX_STROKES used to store the variables. The parameters are the MAX_UNIT_SIZE (The number of characters which the program will take), being 45, header guards to prevent the program being repeated in multiple locations, and all the other header files which was used in the WordIntoGCode.c file.

GetWordLength.c – Contains the prototype and implementation of the GetWordLength function, the function saves the current position of the file and measures the current word length to ensure it is not broken if the line reaches the space limit. The conditional compilations are a declaration for the variable pos (which reads the current position testFile.txt), c (Which contains the character read from the testFile.txt) and length. The parameters are preprocessor directives included in the WordIntoGCode and LoadFontMap files but also included the GetWordLength.h file.

GetWordLength.h – Contains the prototypes and similar parameters from the .c file

EndRobotDrawing.c – Contains the prototype and implementation of the EndRobotDrawing function, this function sends code directly to the robot to return to the (0,0) position and pull the pen up. Its parameters are a COM port (serial.c and .h file, rs232.c and h) and the header file (EndRobotDrawing.h)

EndRobotDrawing.h – Contains the prototypes and similar parameters from the .c file

ProcessTextFile.c – Contains the prototypes and implementations of the ProcessTextFile, which takes the textFile, processes  the individual letters to find the strokes, which using the WordIntoGCode, converts and sends the strokes to GCode to the writer WriterRobot. The conditional compilation are that the lines must not exceed the predefined DRAWING_LIMIT_MM.

ProcessTextFile.h – Contains the prototypes and similar parameters from the .c file

Serial.h and .c – COM port configurations, which sends the G Code to the robot writer

Main.c – Primary operation of code takes place here

Rs232.h and .c – Rs232 library where are G Code information is stored

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| FontData | Struct | Struct which formats the FontMap into first variable marker, being “999”, an ASCII character which would associate with a later in the English alphabet, and a embedded struct which are the number of strokes to write the character called PenData |
| PenData | Struct | Struct which formats the 3rd value in the Font Data into 1st value being the position of the X axis, 2nd value the position of the Y axis and 3rd value the state that the pen is in (0 or 1) |
| userHeight | Int | The user’s individual input of the text height, this will be checked by the program to see if it is between the 4mm to 10mm limit |
| FontMap | Array | Array which will store the SingleStrokeFont file |

Extend table as required

# Functions

int textScale(float textHeight);

Parameters:

textHeight – user’s input for the height of the text

Returning value – A to scale size of the user’s text height

WordIntoGCode(char letter, double textScale, double *currentY, double *current)

Parameters:

Letter – Individual letter from text file

textScale – The value of textScale from the textScale function

currentY – Current position of the pen in the Y axis

currentX – Current position of the pen in the X axis

Returning value – The G Code (“G0 12 25” etc) of the provided letter

LoadFontMap(const char *filename);

Parameters:

Filename – Name of the file provided, in this case it would be “SingleStrokeFont.txt”

Returning value – The FontMap array containing all the strokes for each ASCII character, in the struct of FontData

GetWordLength (GET *filename, double scale);

Parameters:

Filename – Contains the name of the file, in this “testFile.txt”

Scale – Contains the scale value from the textScale Function

Returning value – The word length as a integer

ProcessTextFile (const char *filename, double userHeight);

Parameters:

Filename – Name of the file, in this case “testText.txt”

userHeight – The user’s input of text height

Returning value – G Code of the full message from the “testText.txt”

EndDrawingRobot ();

Parameters:

Returning value – Sends G Code to the robot which tells it to move back to (0,0) and move the pen up

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| textScale | textScale(float textHeight); | True – textHeight = 6.5 False – TextHeight = 122 | This value of textHeight will return a value of scale because it is between the 4 to 10 mm limit This value will return an error message stating that the user input is not within the limit, and so the function will default to 10mm. |
| WordIntoGCode | WordIntoGCode(char letter, double textScale, double *currentY, double *current) | True – letter = T, textScale = 0.444, Current Y = 0.0, Current X = 0.0 False – letter = R , textScale = 0.44, CurrentY = W, Current X = 5 | These values will output Gcode for the letter “T” CurrentX and Y are not doubles, but chars, therefore there would be no coordinates to offset from, giving an error |
| LoadFontMap | LoadFontMap(const char *filename) | True – filename = “SingleStrokeFont.txt” False – filename = “testFile.txt” | This value would open the correct file and load it into the FontMap As this is not the correct file to open, the function would automatically output an error message |
| GetWordLength | GetWordLength (GET *filename, double scale) | True – filename = “testFile.txt”, scale = 0.44 False – filename = “testFile.txt”, scale = Van | Values are correct, so the function would output a length for each word The scale is not an integer, therefore the function cannot scale the lengths to the user’s height and so will fail to run |
| ProcessTextFile | ProcessTextFile (const char *filename, double userHeight); | True – filename = “testFile.txt” userHeight = 9.0 False – filename = “SingleStrokeFont.txt” userHeight = 9.0 | Values are correct so will out G Code for all letters in the corresponding file For the function to complete full, there needs to be characters for it to convert to G Code using the “WordIntoGCode” function, therefore nothing will happen |

# AI Statement

AI was used to understand the project objectives and functions, AI was NOT used to produce code or functions

# Flowchart(s)

Included in zip file as a PDF